Barcode Reader Specification

SICK - CLV632-0120

Customer:	Vistaprint Winterthur

Project:		Line ALADDIN Paper Stream

Projectnr..:	10116.04.01

Version:		0.01

Storage:	D:\daten\60_Projekte\10116 Vista Print\04 Line - ALADDIN\01 Development\20_Engineering\20_Development\Specification\Line-Aladdin Shirtstream PLC Spec V0.12.docx

# Table of contents

1	Table of contents	2

2	Introduction	3

2.1	Scope	3

2.2	Reference Documentation	3

2.3	Version	3

2.4	Abbreviations, definitions, glossary	3

3	Common information’s	4

3.1	Used software components and libraries	4

3.2	Used SOPAS commands	4

3.3	Test environment	4

3.4	Sensor settings (Object trigger)	5

3.5	Using the terminal emulator	5

4	Controller Description	6

4.1	Level 0 Element:  Barcode controller (Class BCC)	6

4.2	Driver Element:  Barcode driver (Class BCD)	8

# Introduction

## Scope

This document gives an overview about the functions of controller BCC and BCD for controlling the SICK barcode reader and is solely for the benefit of Vistaprint and all the persons that are involved at platform development. These are software developers, quality engineers and maintenance engineers.

## Reference Documentation

| Documents | Version | Datum |
| --- | --- | --- |
| CLV62x-65x-Client-V510_ProgrammingGuide.pdf | V5.10 | 2011-08-19 |
| CLV63x_manual.pdf |  | 2008-04-16 |
| SICK Application Report - CLV6xx_Commands.doc |  | 2011-01-28 |
| Beckhoff InfoSys - TF6310 TC3 TCP/IP |  |  |

## Version

| Description | Author | State | Version | Date |
| --- | --- | --- | --- | --- |
| Start | AVME/DSC | d | 0.1 | 2014-01-09 |

State: d = design, r = released

## Abbreviations, definitions, glossary

| Designation | Name |
| --- | --- |
| SOPAS | Software engineering tool for configuration of SICK components |

# Common information’s

## Used software components and libraries

| Description | Version | Datum |
| --- | --- | --- |
| SOPAS engineering tool | V2.38.0 Build 5712 |  |
| Beckhoff TCP/IP server (TF6310-TCP-IP) | V1.0.66.0 | 2013-08-07 |
| TwinCAT TCP/IP connection server library (Tc2_TcpIp) | V3.3.1.0 |  |

## Used SOPAS commands

| Command | Description |
| --- | --- |
| sRI0 | Communication test. |
| sMN mTCgateon | Command to trigger the scanner. (Type of SOPAS method) |

## Test environment

## Sensor settings (Object trigger)

The following settings are necessary that the sensor can listen to SOPAS commands send over TCP/IP.

Start/Stop over command. Duration specifies the sampling. (Time in that the sensor waits for a barcode)

## Using the terminal emulator

For sending commands to the device, the SOPAS integrated terminal emulator can be used.

The following example shows how the trigger can be started to read a barcode by sending command

sMN mTCgateon.

# Controller Description

## Level 0 Element:  Barcode controller (Class BCC)

Functionality

The barcode controller handle the communication to the barcode reader driver.

Main Sequence

Commands

- PON 	(Open the TCP socket and execute a communication test)

- OFF 	(No Action Done)

- READ	(Read barcode. Command switch on the sensor gate for X-time)

Configuration

| Variable | Datatype | Description |
| --- | --- | --- |
| strSensorIPAdr | STRING(15) | IP Adress of the sensor (PLC is the client) |
| udiSensorPort | UDINT | Port number of the sensor |

Parameter

| Variable | Datatype | Description |
| --- | --- | --- |
| udiTOPowerOn_ms | UDINT | [ms] Timeout power on |
| udiTORead_ms | UDINT | [ms] Timeout read |

Current Values

| Variable | Datatype | Description |
| --- | --- | --- |
| bolConnected | BOOL | Sensor connected |
| bolComChecked | BOOL | Communication checked during PON |
| bolNewCodeReaded | BOOL | 1 = new bar code readed, 0 = *No Read* |
| stData | T_BCC_Data | Sensor data |

Device data T_BCC_Data

| Variable | Datatype | Description |
| --- | --- | --- |
| uliBarCode | ULINT | The read barcode |
| strBarCode | STRING | The read barcode. If no barcode has been read ‘NoRead’ |

Errors

- Communication error between PLC and barcode reader			category: INFO

- Initialisation error from barcode reader. No answer from device.		category: OFF

- Barcode could not be read. Answer *NoRead* from device 			category: INFO

- Timeout wait on data 							category: INFO

IO

- No physical input/outputs

## Driver Element:  Barcode driver (Class BCD)

Functionality

TCP-IP driver for SICK barcode reader Type “CLV632-0120”. This driver work as a TCP client.

Main Sequence

Data Handling

Commands

- CONNECT_SOCKET 	(Connect TCP socket)

- CLOSE_SOCKET		(Close TCP socket)

- COM_TEST		(Execute a communication test. Sending CMD and check answer)

- READ_BARCODE		(Read barcode and prepare data)

- READ_DEV_INFO	(not yet integrated)

- READ_OP_DATA		(not yet integrated)

- READ_SERVICE_INFO	(not yet integrated)

Configuration

| Variable | Datatype | Description |
| --- | --- | --- |
| bolSimulation | BOOL | Not used |
| strSensorIPAdr | STRING(15) | IP Adress of the sensor (PLC is the client) |
| udiSensorPort | UDINT | Port number of the sensor |

Parameter

| Variable | Datatype | Description |
| --- | --- | --- |
| tTOConnection | TIME | Default TO for FB_ClientServerConnection |
| tTOSend | TIME | Default TO for FB_SocketSend |
| tTOReceive | TIME | Default TO for FB_SocketReceive |

Device data T_BCC_Data

| Variable | Datatype | Description |
| --- | --- | --- |
| bolCodeReaded | BOOL |  |
| uliBarCode | ULINT | The read barcode |
| strBarCode | STRING | The read barcode. If no barcode has been read ‘NoRead’ |
| stDevInfo | T_BCD_DeviceInfo | Device Information |
| stServInfo | T_BCD_ServiceInfo | not currently used |
| stOpData | T_BCD_OperatingData | not currently used |

T_BCD_DeviceInfo

| Variable | Datatype | Description |
| --- | --- | --- |
| strComTestResult | STRING(40) | Contains results after communication test |
| strManufacturer | STRING(25) | not currently used |
| strDeviceType | STRING(25) | not currently used |
| strOrderNumber | STRING(20) | not currently used |
| strSWVersion | STRING(10) | not currently used |
| strFWVersion | STRING(10) | not currently used |
| strSerialNumber | STRING(20) | not currently used |

T_BCD_OperatingData

| Variable | Datatype | Description |
| --- | --- | --- |
| strPowerOnCounter | STRING(25) | not currently used |
| strOpHours | STRING(25) | not currently used |
| strPowerOnHours | STRING(20) | not currently used |

T_BCD_ServiceInfo

| Variable | Datatype | Description |
| --- | --- | --- |
| strLastUsername | STRING(25) | not currently used |
| strLastParChangeDate | STRING(10) | not currently used |
| strLastParChangeTime | STRING(5) | not currently used |